TASK 1 :

1.1. Power Disconnection & Anti-Static Safety:Safety Isolation.

- Action: Shut down the PC completely, flip the PSU switch to OFF (0), and unplug the AC power cable. Pressed the front power button twice to drain residual capacitance.

- Safety Check: Wore an anti-static wrist strap anchored to an unpainted metal chassis frame to prevent Electrostatic Discharge (ESD) damage to sensitive DRAM ICs.

2.2. Unlatching and Removal:Physical Disengagement.

- Action: Opened the left side panel. Pushed down on the retention latches at the ends of DIMM Slot A2.

- Observation: On modern single-latch slots, only the top latch opens. Firmly gripped the RAM stick by its heat spreader edges and lifted straight up out of the slot.

3.3. Inspection & Re-insertion:Cleaning & Alignment.

- Action: Inspected gold edge contacts for oxidation or debris; cleaned with 99% Isopropyl Alcohol and a lint-free cloth. Aligned the mechanical key notch on the gold contacts with the raised plastic key inside the DIMM slot.

- Observation: Misaligning the notch prevents the module from seating. Reversed orientation ensures correct polarity.

4.4. Reseating & System Verification:Locking Engagement.

- Action: Applied firm, even downward pressure on both top and bottom edges of the RAM stick simultaneously until the plastic retention clips snapped shut automatically.

- Verification: Plugged in power and booted into BIOS. Navigated to the Main tab and confirmed total memory size was correctly recognized (e.g., 16384 MB) running in Dual-Channel Mode.

TASK 2 :

GPU TROUBLESHOOTING FLOWCHART

1. Identify Symptoms    ---> Screen artifacts, driver crashes, black screen under load

2. Physical Inspection  ---> Reseat GPU, check PCIe latch & 8-pin power cables

3. Isolate Display Path ---> Swap HDMI/DP cables & test alternative monitor port

4. Driver Sanitation    ---> Run DDU (Display Driver Uninstaller) in Safe Mode

5. Stress Test & Fix    ---> Run FurMark/3DMark stress test; monitor thermals

- Initial Symptom Identification: Document exact behavior—such as visual artifacts (green/magenta dots, screen tearing), display driver crashes (nvlddmkm.sys TDR error), black screens when launching 3D games, or GPU fans spinning at 100% with no signal.

- Physical Connection & Power Integrity Check: Shut down the PC, re-seat the graphics card firmly in the top PCIe x16 slot until the latch clicks, and verify that dedicated 8-pin / 12VHPWR PCIe power cables are completely flush with no gaps.

- Display Path & Output Isolation: Switch from DisplayPort to HDMI (or vice versa), test a secondary monitor, and rule out bad display cables. If the CPU has integrated graphics (iGPU), connect the monitor directly to the motherboard I/O to confirm the rest of the system boots normally.

- Software & Driver Sanitation (DDU Protocol): Boot Windows into Safe Mode, run Display Driver Uninstaller (DDU) to completely wipe old NVIDIA/AMD display drivers and registry keys, reboot, and perform a clean installation of the latest WHQL display driver.

- Stress Testing & Fix Confirmation: Launch a GPU stress test (e.g., FurMark or 3DMark Time Spy) for 30 minutes while monitoring GPU Core and Hotspot temperatures in HWMonitor. A successful test with no crashes, thermal throttling (>85∘C), or artifacts confirms the fix.

TASK 3 :

Storage Drive Disconnection Simulation & BIOS Indicators

Stage A: Drive Unplugged (Simulated Storage Failure)

- BIOS / UEFI Indicators:

  - Under SATA Configuration / NVMe Port Test, the corresponding slot displays Not Present, Empty, or N/A.

  - The Boot Priority menu removes the drive from the boot list entirely.

- POST / OS Error Screen:

  - The system fails to boot and drops to a black screen displaying:

Reboot and Select proper Boot device or Insert Boot Media in selected Boot device and press a key

(or enters UEFI Setup automatically).

Stage B: Drive Reconnected (Restored System)

- BIOS / UEFI Indicators:

  - The storage controller detects the drive hardware immediately (e.g., Samsung SSD 980 PRO 1TB visible in slot M.2_1).

  - Windows Boot Manager reappears as the primary entry in the Boot Option Priorities menu.

- POST / OS Behavior:

  - The PC bypasses POST warnings and boots cleanly directly into the Windows desktop environment.

TASK 4 :

Random PC Restarts: Primary Component Selection & Diagnostics

Component Checked First: RAM (System Memory)

Technical Reasoning

When a PC restarts randomly (without a Blue Screen or warning), the underlying cause is frequently unstable or failing RAM cells. When the CPU attempts to read from or write to an invalid address in faulty memory, a kernel-level hardware exception occurs (KMODE_EXCEPTION_NOT_HANDLED or PAGE_FAULT_IN_NONPAGED_AREA).

Because Windows cannot safely recover from a corrupted kernel stack, the OS immediately triggers a hard reset to protect data integrity. Furthermore, RAM issues are fast to test and isolate without specialized spare hardware. (Note: If restarts occur exclusively under heavy 3D load, the Power Supply Unit is the second most likely culprit).

MEMORY DIAGNOSTIC TEST FLOW

1. Windows Memory Diagnostic ---> Quick built-in software memory sweep

2. MemTest86 USB Boot        ---> Multi-pass deep bit-level pattern test

3. Single-Stick Isolation     ---> Test one RAM module at a time in Slot A2

4. XMP / EXPO Adjustments     ---> Disable aggressive memory overclocks

Diagnostic Steps for RAM

- Disable Automatic Restart in Windows:

In Windows, open sysdm.cpl → Advanced → Startup and Recovery Settings → Uncheck "Automatically restart". This forces the PC to display the BSOD error code instead of abruptly rebooting.

- Run Built-In Windows Memory Diagnostic:

Type mdsched.exe in the Windows search bar, select Restart now and check for problems, and observe if hardware errors are flagged on the initial pass.

- Run Standalone MemTest86 (Gold Standard):

Create a bootable USB drive with MemTest86, boot directly into it from BIOS, and run a minimum of 4 complete passes of deep bit-pattern testing. Any result above 0 errors indicates a hardware fault.

- Isolate Individual Sticks:

If errors appear, remove all RAM sticks and test one module at a time in slot A2. Repeat MemTest86 on each module individually to identify whether a single stick is bad or if the memory profile (XMP/EXPO) requires lowering frequency/tightening voltage.